EXPERIMENT NUMBER : 1

FAMILIARIZATION OF PASSIVE COMPONENTS

EX: 101

Fundamentals of electrical and electronics

INSTRUCTORS:  Devendra Kathyal

: Dakshina Shrestha

AUTHORS :  Prabodh Paudel

: Pranish Kharel

:  Pratik Pandey

: Shreeya KC

EXPERIMENT CONDUCTED ON : 2082/08/10

REPORT SUBMITTED ON : 2082/08/17

Department of electronics and computer

Engineering

SAGARMATHA ENGINEERING COLLEGE

Sanepa-2, Lalitpur

Objective:

The main objective of this experiment is to gain familiarization with passive electronic components i.e resistors and capacitors. This includes learning to identify resistor values using color coding, understanding the significance values of different color bands, tolerance and temperature coefficient of resistors. The experiment also aims to distinguish between four-band, five-band, and six-band resistors and accurately calculate resistance values using the color code formula . Additionally, the objective extends in measuring the resistance using a multimeter and compare theoretical and practical values. Similarly, it also helps in identifying and determining the capacitance value of a capacitor through direct reading and calculation.

Background Theory / Introduction

Passive component are those component which donot require specific voltage to operate. Instead, they absorb, store, or dissipate energy. They are essential to ensure proper functioning, signal control, energy management, and stability.Resistor and capacitor are some passive components among them.

A resistor is a passive component that opposes the flow of electric current. Since most resistors are very small in size, their resistance values are represented using a color code system. The resistance value, tolerance, and  its temperature coefficient is determined by the color coding of the color band printed in the resistors.  By interpreting these color bands, the theoretical resistance of the resistor can be calculated. Resistors can have four, five, or six color bands, depending on the level of accuracy and type of application.

- 4-band resistor: It is used in common everyday electronic devices.

- 5-band resistor: It is used where more accurate resistance is needed.

- 6-band resistor: It is used  in advanced circuits that need high accuracy and temperature stability.

The significant value, Tolerance % and temperature coefficient for each colors are:

| Color | Significant Value | Tolerance % | Temperature Coefficient |
| --- | --- | --- | --- |
| Black | 0 | - | 250 |
| Brown | 1 | 1 | 100 |
| Red | 2 | 2 | 50 |
| Orange | 3 | - | 15 |
| Yellow | 4 | - | 25 |
| Green | 5 | 0.5 | 20 |
| Blue | 6 | 0.25 | 10 |
| Violet | 7 | 0.1 | 5 |
| Grey | 8 | 0.05 | 1 |
| White | 9 | - | - |
| Gold | 10-1 | 5 | - |
| Silver | 10-2 | 10 | - |
| No Color | - | 20 | - |

Fig:four band resistor

(Source:google)

A capacitor is another passive component that stores electrical energy in the form of an electric field. There are two types of capacitors i.e electrolytic(having polarities) and non electrolytic (which donot have polarities) . It is widely used for energy storage. Capacitors usually display their capacitance value directly on their body through numeric codes or printed labels.

Components Used

In this experiment, the main component used was a color coded resistor.  Five resistances were provided, each with a set of color bands that indicate their resistance value, tolerance, and temperature coefficient. Both four band resistors and five band resistors were used.

A digital multimeter (ohmmeter) was also used to measure the actual resistance of each resistor. This allowed comparison between the calculated theoretical resistance and the measured resistance. Additional components used included three types of capacitors, which were part of the component to be familiarized with.

Equipment and Components

A. Digital Multimeter / Ohmmeter

- Brand: UNI-T

- Model Number: UT39A+

B. Color Coded Resistors

C. Capacitors (Three Types Used)

Part 1: Measurement of Resistance by the Use of Color Coding:

Methods:

- Resistor Color Coding

Resistor use colored bands to indicate their resistance value. The meanings differ for 4-band and 5-band resistors.

A. Formula for 4-Band Resistor

R=A×B×10C±D%

Where:

- A = 1st significant digit

- B = 2nd significant digit

- C = Multiplier

- D = Tolerance

B. Formula for 5-Band Resistor

R=A×B×C×10D±E%,F

Where:

- A = 1st significant digit

- B = 2nd significant digit

- C = 3rd significant digit

- D = Multiplier

- E = Tolerance

- F = Temperature Coefficient

Procedure:

Step 1: Identify Color Bands

- Observe the color bands on each resistor.

- Write the colors in order from left to right.

- Refer to the color code chart to find:

  - Significant digits

  - Multiplier

  - Tolerance

  - Temperature coefficient (if given)

Step 2: Calculate Resistance

For 4-band resistors, apply
            R=A×B×10C±D%

For 5-band resistors, apply

R=A×B×C×10D±E%,F

Calculate tolerance range:

  - Find Rmin and Rmax

Record all values in the observation table.

Step 3: Measure Using Ohmmeter

- Turn on the multimeter and set it to resistance (Ω) mode.

- Connect the probes to the resistor leads.

- Wait for the reading to stabilize.

- Set the resistance range to nearest highest on DMM.

- Record the measured resistance value.

Step 4: Compare Values

- Check if the measured resistance lies within the tolerance range:

- If inside the range → Resistor is correct

- If outside the range → Resistor is faulty or out of tolerance

Observation:

Table 1: Resistance value of the resistors using multimeter

| S.N | Color | Calculate Resistance (Ω) | Resistor Range | Measured Resistance (Ω) | Error %=Rc-RmRc |
| --- | --- | --- | --- | --- | --- |
| 1 | Red, Violet, Black, Gold | 27 | 25.65-28.35 | 26.5 | 1.85% |
| 2 | Yellow, Violet, Black, Red, Brown | 47000 | 46530-47470 | 46900 | 0.21% |
| 3 | Blue, Grey, Brown, Gold | 680 | 646-714 | 668 | 1.76% |
| 4 | Brown, Orange, Black, Green, Brown | 150000 | 151500-148500 | 149600 | 0.26% |
| 5 | Yellow, Violet, Red, Gold | 4700 | 4465-4935 | 4610 | 1.91% |

Experiment part 2:- Measurement of capacitance.

Method and Procedure:

For this process of measurement of capacitance, we used 3 different capacitors which can be written into two types:

- Electrolytic capacitor (Aluminium electrolytic).

- Non-Electrolytic capacitor:  Polyester Film Capacitor/Mylar capacitor and Ceramic disc capacitor.

A digital multimeter similar to that of the experiment in the measurement of resistance with the same connection of the wires.

The multimeter in this process was set in to capacitance mode by rotating the dial at the middle. The 3 different capacitors were first discharged by shorting which helped us to see the steady increase in the capacitance value of the capacitor.

Calculation formula for measurement of capacitance:

Capacitance (C)= AB x 10ᶜ x 10-12 F

Observation table:

| S.N | Number | Calculated value (μF) |
| --- | --- | --- |
| 1 | 224 | 0.22 μF |
| 2 | 103 | 0.01 μF |
| 3 | 102 | 0.001 μF |

Here, for first capacitor,

A=2, B=2 and C=4

Then, Capacitance (C)= AB x 10ᶜ x 10-12 F

Or, C= 22 x 104 x 10-12F = 2.2 x 10-7F = 2.2 μF

Similarly,

For second capacitor,

A=1, B=0 and C=3

So, C= 10 x 103 x 10-12f = 10-8F = 0.01 μF

And,

For third capacitor,

A=1, B=0 and C=2

So, C= 10 x 102 x 10-12F = 10-9F = 0.001 μF

OBSERVATION:

We used the color coding in the resistor and calculated the value of resistance using multimeter.

We used the value written in capacitor to determine the capacitance of the capacitor using multimeter.

The multimeter showing value close to the required range/value was observed in both capacitor and resistor.

We had to make sure that the capacitor had been discharged by shorting the two terminals of the capacitor.

For resistor we made sure that the color coding was good and had to make sure that the value was not unstable and inconsistent. Whereas for capacitor, the gradually increasing value indicated that the capacitor was now charging slowly.

Findings and Result:

- The theoretical resistance values calculated from the color codes were very close to the multimeter readings. All measured values stayed within the tolerance range, proving the resistors were accurate and functioning properly.

- For capacitors, the calculated values from the number codes matched the multimeter readings fairly well. The electrolytic capacitor showed a slow rise in reading due to charging, which is normal.

- Overall, both resistance and capacitance measurements were consistent and verified the correctness of the components.

Data Discussion:

- The resistor data shows that color coding is a reliable method for identifying resistance values, and the multimeter readings supported this. Small differences between calculated and measured values happened mainly due to tolerance limits and measurement conditions.

- Capacitor data was also consistent. Ceramic and film capacitors gave stable readings, while the electrolytic capacitor behaved as expected by slowly charging. The values being close to the calculated ones indicates the components were in good condition.

- The experiment successfully demonstrated the relationship between theoretical values and practical measurements.

Error Analysis:

- Slight variations in resistance readings may have occurred because of probe contact issues, human reading error, meter accuracy, or environmental factors.

- Error percentage was calculated by using the formula:  Error Percentage=Rc-RmRc*100

- Capacitor readings may have differed slightly due to incomplete discharge, internal leakage, or multimeter sensitivity.

- Coding errors (misreading color bands or capacitor numbers) could also introduce small calculation mistakes.

- Overall errors were minor and did not affect the final outcome significantly.

Problems / Questions:

- Distinguishing between some resistor color bands (like red–brown or violet–grey) was sometimes confusing.

- The electrolytic capacitor needed to be discharged every time, otherwise the readings fluctuated.

- Multimeter settling time for capacitance measurement took longer than expected.

- A question that came up was whether the tolerance band always guarantees the measured value will fall inside the range, which the experiment confirmed.

Recommendations and Conclusions:

### Recommendations:

- Double-check resistor colors under good lighting to avoid misreading.

- Always discharge capacitors before connecting them to the multimeter.

- Handle the multimeter probes steadily to avoid unstable readings.

- Use auto-range mode when available to reduce manual mistakes.

### Conclusions:

- The experiment successfully met its objective of familiarizing us with passive components.

- Both resistors and capacitors showed readings very close to their theoretical values, proving that the components were accurate and the measurement process was correctly performed.

- This experiment improved our understanding of how passive components behave and how to measure them properly in real circuits.

References:

- byjus.com

- neurophysics.ucsd.edu

- https://www.electronics-tutorials.ws/resistor/res_2.html

- https://www.electronics-tutorials.ws/capacitor/cap_1.html