Experiment part 2:- Measurement of capacitance.

Procedure:

For this process of measurement of capacitance, we used 3 different type of capacitors and a digital multimeter similar to that of the experiment in the measurement of resistance with the same connection of the wires.

The multimeter in this process was set in to capacitance mode by rotating the dial at the middle. The 3 different capacitors were first shorted which helped us to take the measurement accurately and protect the apparatus used in this experiment.

The capacitor terminals were then connected to the multimeter cables, allowed the reading to be stabilized and then the reading was finally noted.

Calculation formula for measurement of capacitance:

Capacitance (C)= AB x 10ᶜ x 10-12 F

Observation table:

| S.N | Number | Calculated value (μF) |
| --- | --- | --- |
| 1 | 224 | 0.22 μF |
| 2 | 103 | 0.01 μF |
| 3 | 102 | 0.001 μF |

Here, for first capacitor,

A=2, B=2 and C=4

Then, Capacitance (C)= AB x 10ᶜ x 10-12 F

Or, C= 22 x 104 x 10-12F = 2.2 x 10-7F = 2.2 μF

Similarly,

For second capacitor,

A=1, B=0 and C=3

So, C= 10 x 103 x 10-12f = 10-8F = 0.01 μF

And,

For third capacitor,

A=1, B=0 and C=2

So, C= 10 x 102 x 10-12F = 10-9F = 0.001 μF